ERCOT 2008 SEVERE WEATHER DRILL SCRIPT

# DRILL TITLE:

# Event Caused by Category 4 Hurricane

DRILL PURPOSE:

The ERCOT 2008 Severe Weather Drill is to provide TDSP, QSE, and ERCOT Operators with simulated emergency operations practice.  It is anticipated that each operator participating in the Drill will:

- Practice communicating with other entities not only in events leading up to the Hurricane event (on May 12, and May 13), but also appropriate restoration activities on the day of the Drill (on May 14)

- Have the opportunity to simulate the use of internal backup emergency plans

- Have the post-Drill opportunity to determine lessons learned, to suggest needed changes to emergency and emergency operations plans, and to critique the Drill

- Have the opportunity to exercise other emergency centers within your organization.

- Have the opportunity to meet NERC & ERCOT Guides’ requirements for emergency training

DRILL SCENARIO:

(The drill will compress a 9-day event into a 3-day drill period)

On Day 1, ERCOT receives a weather advisory on a low pressure disturbance in the Yucatan Peninsula.

Day 1 Drill Coordinates…

Latitude N 22.3/Longitude W 86.3/Wind 30MPH/Gusts 35MPH

By Day 2, the disturbance – Southern Gulf of Mexico – has been upgraded to a tropical depression.

Day 2 Drill Coordinates…

Latitude N 22.5/Longitude W 89.3/Wind 40MPH/Gusts 45MPH

By Day 3, ERCOT receives a forecast advising that the tropical depression has been upgraded to a named tropical storm, and is fast developing in the southern Gulf of Mexico.  Meanwhile, offshore production facilities have already initiated shut-in and evacuation procedures, causing steep fuel curtailments that are likely to impact generation availability.

Day 3 Drill Coordinates…

Latitude N 23.0/Longitude W 92.1/Wind 45MPH/Gusts 55MPH

Over the weekend of Day 4 and Day 5, the National Hurricane Center in Miami conducts two storm tracker reconnaissance flights.  By Day 5, the National Hurricane Center forecasts a Category 4 or 5 hurricane, with projected landfall along the lower to middle Texas coast within 72-96 hours, between Day 8 and Day 9.  Given current coordinates and landfall projections, county and local authorities in Texas strongly advise residents in low-lying coastal areas to begin immediate evacuation preparations, before mid-week.

Day 4 Drill Coordinates…

Latitude N 23.5/Longitude W 93.4/Wind 75MPH/Gusts 90MPH

Day 5 Drill Coordinates…

Latitude N 24.1/Longitude W 94.5/Wind 115MPH/Gusts 130MPH

Day 6, 0900/OPERATING CONDITION NOTICE

(0900) Drill Time, May 12

Drill Notes…

Updated forecasts Monday morning – Day 6 – now point to a landfall on Day 8, along the central Texas coast, with sustained winds exceeding 131 mph.  ERCOT also anticipates a severe shortfall of load, such that insecure system conditions are projected.  ERCOT responds to that by issuing an Operating Condition Notice, due to forecasted weather and system loads.

Day 6 Drill Coordinates…

Latitude N 24.5/Longitude W 95.4/Wind 125MPH/Gusts 155MPH

Drill Action Items…

- ERCOT calls TOs/TDSPs and QSEs, advising of the potential for adverse reliability impacts, and the need for contingency preparations

- ERCOT advises TOs/TDSPs to review Planned Outages to determine if they should be cancelled or if current outages should be restored prior to projected landfall and report status to ERCOT.

- ERCOT instructs QSEs to review fuel supplies, resource plans, resource maintenance/outages…and report status back to ERCOT, including submitting Fuel Report Form(s) to ERCOT.

- ERCOT coordinates with NRG on STP’s mandatory emergency shutdown procedures (as required by NRC)

- All entities are to review internal preparations (to include possible staff evacuations to Backup Control Center facilities)

- Any entities relocating personnel to backup control centers are to immediately notify ERCOT, and make preparations to test backup communications and systems between backup centers and ERCOT.

DAY 6, 1830/ADVISORY

(1430 Drill Time, May 12)

Drill Notes…

By 1800 Day 6, the National Hurricane Center issues an advisory for hurricane conditions for Day 8.

Hurricane is expected to approach South Padre Island and move North- easterly along the Texas Gulf Coast, eventually making landfall in or around the San Antonio bay area.  Meanwhile, further review of load and resource forecasts indicates to ERCOT that loads could be significantly lower, and resource availability could be significantly greater (than what is required).

ERCOT issues an Advisory at 1830 on Day 6 to all TOs and QSEs, reporting that conditions are developing that will require as much down balancing energy service (DBES) as possible.  ERCOT also instructs Market Participants to initiate internal storm response procedures, to include voluntary relocation to Backup Control Centers.

Drill Action Items…

- ERCOT instructs all entities to review internal preparations (to include possible staff evacuations to backup control center facilities)

- TOs and QSEs to report to ERCOT on status of operations (to include status of primary/backup control centers, and any staffing changes at primary/backup control centers, due to evacuations)

- Any entities relocating personnel to backup control centers are to immediately notify ERCOT, and make preparations to test failover communications/systems between Backup Control Centers and ERCOT

- ERCOT instructs TOs/TDSPs to restore any remaining outaged equipment able to be returned to service…then report equipment status back to ERCOT

- ERCOT instructs TOs/TDSPs to file/update DOE Form 417.

DOE Form 417:  ftp://ftp.eia.doe.gov/pub/electricity/eiafor417.doc

- ERCOT instructs QSEs to notify all resources of system conditions

- ERCOT instructs QSEs to make available any outaged resources able to be returned to service…then report equipment status to ERCOT

DAY 7, 1300 / ALERT

(1300 Drill Time, May 13)

Drill Notes…

Storm conditions and projections have not changed throughout the day on Day 7.  Meanwhile, ERCOT has reviewed the feedback reported from TOs and QSEs, and has determined that significant load will be lost in the coastal area.  Separately, ERCOT determines the need to inventory resource availability for both voltage control and restoration purposes.  ERCOT prepares for an emergency, as it is imminent that within 18-24 hours, ERCOT will be in an emergency operating condition.

ERCOT issues an Alert at 1300 on Day 7 to all TOs and QSEs, requesting information as to resource availability (for voltage control and restoration purposes).  Additional DBES will be procured overnight, in advance of landfall.

Day 7 Drill Coordinates…

Latitude N 25.2/Longitude W 96.2/Wind 135MPH/Gusts 170MPH

Drill Action Items…

- ERCOT instructs QSEs to bid in all available DBES, adjusting Resource Plans, as appropriate

- ERCOT instructs QSEs to verify latest storm impact on fuel availability…then report status back to ERCOT, including submitting Fuel Reporting Form(s) to ERCOT

DAY 8, 0800/COMMUNICATION TESTING

(0800 Drill Time, May 14)

0800:  Test all communications systems between participating entities; this includes telephones, satellite phones, faxes, radios, internet communications, backup control centers, hotlines, and etc.

ERCOT will test the SSRG Hotline and initiate a Hotline Test Call. Call will be made at 08:30 SSRG roll-call at 08:40

DAY 8, 0900/LANDFALL & EMERGENCY NOTICE

(0900 Drill Time, May 14)

Drill Notes…

The Hurricane makes impact on Day 8 at ~0900 at South Padre Island as projected.  High winds and heavy rain are occurring between Brownsville and Corpus Christi.  Storm surges of 8-14 feet are reported.  Separately, the latest reports by QSEs on fuel availability lead ERCOT to conclude that severe fuel supply shortages will exist during and following the event.

Day 8 Drill Coordinates…

Latitude N 26.2/Longitude W 96.4/Wind 150MPH/Gusts 180MPH

ERCOT issues an Emergency Notice at 0900 to all TOs and QSEs, due to weather conditions, high voltage, and fuel curtailments.

Drill Action Items…

- ERCOT instructs TOs/TDSPs to prepare to lose load and expect high voltage conditions

- ERCOT instructs QSEs to be prepared to reduce output, due to anticipated loss of load, and to provide necessary reactive power support

- Certain QSEs have been curtailed by their fuel suppliers…these QSEs are to regularly update ERCOT as to fuel and resource status

- ERCOT instructs QSEs to adjust Resource Plans – as pertinent – to reflect impact of fuel curtailments on generation availability

DAY 8, 1000/LANDFALL & SEVERE WINDS

(1000 Drill Time)

Drill Notes…

Hurricane winds are gusting up to 180 mph, and there are unconfirmed reports of associated severe tornadic activity.  TOs/TDSPs report momentary transmission line operations to ERCOT, as well as some severe isolated forced outages.  ERCOT’s reserves currently are adequate, as load is decreasing rapidly.  Communications/telemetry between certain QSEs and ERCOT have been lost, forcing those QSEs’ resources to communicate/operate directly with ERCOT and/or local TOs/TDSPs.

Drill Action Items…

- QSE-resources that have disconnected from their QSEs are to immediately report status to ERCOT and/or local TOs/TDSPs, and to await further instruction

DAY 8, 1200/TORNADOES & TRANSMISSION/SUB-STATION DAMAGE

(1015 Drill Time)

Drill Notes…

The Hurricane has stalled over Central Gulf Coast for the last three hours.  TDSPs/TOs are reporting lines down, due to winds and unknown numbers of tornadoes.  Line crews along the coast have evacuated and assembled inland, but are presently unable to respond, as travel is extremely dangerous.  Support personnel are severely limited ERCOT-wide (at TDSPs/TOs, Generation Resources, and QSE Control Centers), due to massive evacuations the preceding two-three days.

Drill Action Items…

- TDSPs/TOs observing line and substation outages should immediately report status to ERCOT

- QSEs observing any changes to Resource operations, including equipment damage incurred, should immediately report status to ERCOT

DAY 8, 1400/INLAND MOVEMENT

(1045 Drill Time)

Drill Notes…

The Hurricane is now moving inland at 20 mph on a northerly path, forecasted to move just east of Austin, then directly into Dallas-Fort Worth.  Additional tornadoes have been reported.

Day 8 Drill Coordinates…

Latitude N 28.1/Longitude W 96.5/Wind 80MPH/Gusts 120MPH

TOs/TDSPs confirm widespread transmission line and substation damage, as well as other transmission lines that are galloping.  A major transmission line has been forced out in the Central Texas area.  Momentary operations on transmission lines are turning up in several areas.  TOs/TDSPs are reporting that some of these lines may have to be forced out.

Communications/telemetry between additional QSEs and ERCOT have been lost, forcing those QSEs’ resources to communicate/operate directly with ERCOT and/or local TOs/TDSPs.

Drill Action Items…

- TOs experiencing momentary line operations and forced line outages should report status back to ERCOT

- QSEs observing any changes to Resource operations, including equipment damage incurred, should immediately report status back to ERCOT

- QSE-Resources that have disconnected from their QSEs are to immediately report status to ERCOT and/or local TOs/TDSPs, and to await further instruction

DAY 8, 1600/INLAND MOVEMENT

(1100 Drill Time)

Drill Notes…

Load has been lost along the coast, and north through the Austin-Waco corridor, as the Hurricane moves further inland.  The Hurricane has weakened now to a Category-1 storm, as 75-90mph winds gusts are observed.  Reports are now coming in that severe coastal flooding has taken numerous refineries, pipelines, and compressor stations out of service.

TOs/TDSPs report numerous outages on distribution circuits, due to severe winds.  Central Texas has one 345 kv transmission line forced out, along with three 138 kv lines.  South Texas has two 138 kv transmission lines forced out.

Houston-to-North zonal transfers are at the maximum, as online generation attempts to remain online.  Fuel curtailments have continued to increase, causing further generation capability reductions.

Drill Action Items…

- TOs/TDSPs must report status of transmission outage(s) and/or substation damage to ERCOT

- TOs/TDSPs to assess and report customer outages to ERCOT (per DOE Form 417)

- QSEs observing fuel curtailments (and generation reductions) must report status to ERCOT, to include submitting Fuel Reporting Form(s) to ERCOT

- QSEs must update Resource Plans to reflect impact of fuel curtailments

- QSEs observing any changes to resource operations, to include equipment damage incurred, should immediately report status to ERCOT

- QSE-Resources that have disconnected from their QSEs are to immediately report status to ERCOT and/or local TOs/TDSPs, and to await further instruction

DAY 8, 1800/STORM IN CENTRAL TEXAS

(1140 Drill Time)

Drill Notes…

ERCOT determines that certain localized areas are operating in an emergency condition, due to fuel curtailments, dropping load, voltage instability, galloping conductors, additional 69 kv and 138 kv outages.  Widespread line outages are continuing to occur, while other Resource facilities have been isolated due to lack of transmission availability.

Drill Action Items…

- ERCOT deploys additional, available DBES

- ERCOT requests TOs/TDSPs to assess transmission availability at Resource facilities, and report status to ERCOT

- ERCOT requests TOs/TDSPs to assess/report transmission capability

- ERCOT notifies all appropriate QSEs to assess readiness/availability of Blackstart and RMR resources

- QSEs should re-start specific/available resources, as load recovers (per pre-existing bilateral agreements)

- QSEs observing fuel curtailments (and generation reductions) must report status to ERCOT

- QSEs must update Resource Plans to reflect impact of fuel curtailments, equipment failure(s)

- QSE-Resources disconnected from their QSEs are to continue attempting to re-connect with their QSEs, and report status back to ERCOT

- Drill Participants should be practicing internal restoration activities and interacting as appropriate with ERCOT, during this stage of the Drill

Day 8 Drill Coordinates…

Latitude N 31.5/Longitude W 96.4/Wind 40MPH/Gusts 75MPH

DAY 8, 2100/STORM IN NORTH TEXAS

(1330 Drill Time)

Drill Notes…

Severe rain, wind, and flooding are widespread.  Meanwhile, ERCOT’s load is still falling, too much generation remains online, and high voltage remains a problem.  The hurricane has been downgraded to a Tropical Storm, as sustained winds are well below 75 mph now, but it has entered the Dallas-Fort Worth area.  Even as winds have decreased – occasional gusts are exceeding 60 mph – TOs/TDSPs continue to observe galloping conductors.  System load is still falling, and frequency is climbing.

Day 8 Drill Coordinates…

Latitude N 33.2/Longitude W 96.1/Wind 40MPH/Gusts 75MPH

Drill Action Items…

- ERCOT requests TOs/TDSPs to assess and report transmission capability

- TOs/TDSPs to implement Block Load Transfer (BLT) plans, for load restoration, if needed

- QSE-Resources disconnected from their QSEs are to continue attempting to re-connect with their QSEs, and report status back to ERCOT

- Drill Participants should be practicing internal restoration activities and interacting as appropriate with ERCOT, during this stage of the Drill

DAY 8, 2300/STORM IN OKLAHOMA

(1400 Drill Time)

Drill Notes…

Hurricane conditions within Texas have improved, as loss of load has peaked.  However, rain, wind, and flooding continue in most areas.  Winds are now in the 15-20 mph range.  Transmission/distribution outages, coastal infrastructure outages, and certain QSE outages all persist, at this time.  TOs/TDSPs and QSEs are assessing facility damage, while restoration of load continues.

Day 8 Drill Coordinates…

Latitude N 34.4/Longitude W 95.2/Wind 10MPH/Gusts 20MPH

Drill Action Items…

- QSEs re-connected to their Resources are to immediately report status to ERCOT

- Drill Participants should be practicing internal restoration activities and interacting as appropriate with ERCOT, during this stage of the Drill

DAY 8, 2300/HURRICANE CONDITIONS IMPROVING

(1430 Drill Time)

Drill Notes…

ERCOT has received sufficient DBES bids for its reduced loads.  Very few gas curtailments have been lifted, as infrastructure along the coast has sustained extensive damage.  Separately, transmission line/substation damage is massive, but TDSP/TO line crews have begun repair/restoration efforts.

Drill Action Items…

- QSEs should re-start specific/available resources, as load recovers (per pre-existing bilateral agreements)

- QSEs to revise/update RPs, as appropriate

- Drill Participants should be practicing internal restoration activities and interacting as appropriate with ERCOT, during this stage of the Drill

DAY 9, 0200/HURRICANE CONDITIONS IMPROVING

(1500 Drill Time)

Drill Notes…

Additional gas curtailments have been lifted, though there is still undetermined damage to offshore production and coastal infrastructure facilities.

The Central and South Texas transmission lines that were lost (at 1100 Drill Time) have been restored to service.  Damage to other 69 kv and 138 kv lines along the coastal plain will keep these particular lines out of service for a still unspecified period of time.

ERCOT reviews latest Resource and TDSP data, and determines that ERCOT is in a secure state.

Drill Action Items…

- TDSPs/TOs to report status of line outages back to ERCOT

- QSEs instructed to report status of fuel restoration(s) back to ERCOT, and update RPs as appropriate

- Drill Participants should be practicing internal restoration activities and interacting as appropriate with ERCOT, during this stage of the Drill

(2008 SEVERE WEATHER DRILL OVER)

ALL PARTICIPANTS MUST REVIEW AND CRITIQUE DRILL INTERNALLY, THEN PROVIDE WRITTEN critique TO ERCOT BY MAY 16TH.