| Approved by | Title | Signature | Date Approved |
| --- | --- | --- | --- |
| Blevins, Bill | Manager of Operation Support |  |  |

Review Team

| Title | Name | Department |  |
| --- | --- | --- | --- |
| Preparer | Ralph Poston | Outage Coordination |  |
| Manager | Bill Blevins | Operations Support |  |
| Supervisor | Bobby Reed | Outage Coordination |  |
| Reviewer | Darrell Jenkins | Outage Coordination |  |
| Reviewer | Robert Matlock | Outage Coordination |  |
| Choose Title |  | Choose Dept |  |
| Reviewer | Ken McIntyre | Operations & Planning Standards Compliance | Outlook:\\Mailbox - Blevins, Bill\Inbox\ERCOT\Operations\Lunch |

References

| Compliance Reference | Name/Section |
| --- | --- |
| Protocols | Section 3.1.5.1 |
| Choose Reference |  |
| Choose Reference |  |
| Procedure Cross Reference | Procedure Name |
| Choose Reference |  |
| Choose Reference |  |
| Choose Reference |  |
| Document Posting | Location/Landing Page |
| Choose Posting |  |
| Choose Posting |  |
| Choose Posting |  |

Revision History

| Rev # | Preparer | Summary of Changes |
| --- | --- | --- |

## Purpose

This procedure outlines the process for communications between the Resources who own transmission equipment via their designated point of contact to ERCOT in order for ERCOT to submit outage information into the ERCOT Outage Scheduler.

Background:

This procedure establishes a manual process by which a Resource Entity can submit Transmission Equipment Outages on Transmission Equipment owned and operated by them.   This procedure is intended to be a temporary process to be used during the Outage Scheduler enhancement project. The Outage Scheduler enhancement project is expected to create a role for the Resource Entities designated single point of contact (QSEs) to submit outages for Transmission Equipment owned by the Resource Entity.

Until the Outage Scheduler enhancement project is completed and the protocols are revised ERCOT is required to submit and modify Outage Scheduler entries for the Generation Resource for Transmission Elements owned by the Resource.

ERCOT’s obligation to enter outage requests on behalf of the Resource Entities single point of contact is valid only for the purposes of Transmission Facilities Outage Scheduler entry requirements prescribed by the ERCOT Protocols and Operating Guides. With the exceptions mentioned in this document, the established communication procedures listed in the Transmission Outage Approval Process should be followed.

## Procedure

Each Resource Entity that owns Transmission Elements, that is part of the ERCOT Transmission Grid and defined in the Network Operations Model that has been removed from its normal service, excluding the operation of Transmission Facilities associated with the start-up and shutdown of Generation Resources, must submit Planned, Maintenance and Forced outage schedules for ERCOT approval.

Procedure:

- The Resource Entity shall coordinate Transmission Facility outage request with its designated single point of contact (QSE) in accordance with Nodal Protocol 3.1.4.1 Single Point of Contact.

- The Resources Entity’s QSE shall submit outage information to ERCOT staff by email using the “Trans Planned-Maintenance Outage Request Form” or the “Transmission Forced Outage Request Form” depending on the type of outage being requested. The “Trans Planned-Maintenance Outage Request Form” and the “Transmission Forced Outage Request Form” are available on ERCOT.com at http://www.ercot.com/gridinfo/transmission/index .  This information should be emailed to ResTranOutages@ercot.com .

- The QSE shall confirm receipt[1] and review the Outage request with ERCOT Outage Coordination by calling ERCOT Outage Coordination at (512) 248-6842 during normal business hours and calling the ERCOT Shift Engineer at (512) 248-6354 during non-business hours.  Phone Confirmation for forced and Level 1 Maintenance Outage schedules is required.

- ERCOT staff[2] shall enter such requests in the Outage Scheduler on behalf of the Resource Entity as submitted by the QSE.

- The Resource Entity shall monitor the outage information on the ERCOT MIS (https://mis.ercot.com/pps/tibco/mis/Pages/Grid+Information/Transmission ) to ensure that the correct information was entered by ERCOT also to monitor the progression status and identify if the schedule is “Approved” or “Rejected”.

- If the Resource Entity identifies an error has been made, the Resource Entity shall resubmit the correct outage information to their QSE and the QSE shall resubmit the “Trans Planned-Maintenance Outage Request Form” or the “Transmission Forced Outage Request Form” to ERCOT and confirm receipt and review the Outage modifications with ERCOT Outage Coordination by calling ERCOT Outage Coordination at (512) 248-6842 during normal business hours and calling the ERCOT Shift Engineer at (512) 248-6354 during non-business hours.  Phone Confirmation for forced and Level 1 Maintenance Outage schedules is required.

- For Transmission Facilities Outages, ERCOT shall approve or reject each request in accordance with the following table:

| Amount of time between the request for approval of the proposed Outage and the scheduled start date of the proposed Outage: | ERCOT shall approve or reject no later than: |  |
| --- | --- | --- |
| Three days | 1800 hours, two days before the start of the proposed Outage |  |
| Between four and eight days | 1800 hours, three days before the start of the proposed Outage |  |
| Between nine days and 45 days | Four days before the start of the proposed Outage |  |
| Between 46 and 90 days | 30 days before the start of the proposed Outage |  |
| Greater than 90 days | 75 days before the start of the proposed Outage |  |

- If circumstances prevent adherence to these timetables, ERCOT shall discuss the request status and reason for the delay of the approval with the requesting QSE and make reasonable attempts to mitigate the effect of the delay on the Resource Entity.

- If the outage is to be “Rejected”, ERCOT staff will contact the QSE by phone and/or email. ERCOT staff will work with the Resource Entity via their QSE for a solution. If no solution can be reached, the ERCOT staff will notify the QSE by phone and by email that the outage was “Rejected” The message shall identify the contingency and the overloaded element, other known issues and solutions.

- If the outage is “Approved” and the outage starts, the QSE will continue to email ERCOT updated forms to capture the “Actual Start” and “Actual End” times of the Outage.  The ERCOT staff will enter all of the updates into the Outage Scheduler.

- If the outage cannot be completed during the approved time frame, the Resource Entity can submit an updated form to their QSE who will provide this information to ERCOT. ERCOT will enter the information into the Outage Scheduler.  The Resource Entity should monitor ERCOT’s entries to ensure the correct data has been entered by ERCOT. If corrections are required then the Resource Entity should resubmit the correct outage information to their QSE and the QSE shall resubmit the correction and confirm receipt and review the Outage modifications with ERCOT Outage Coordination by calling ERCOT Outage Coordination at (512) 248-6842 during normal business hours and calling the ERCOT Shift Engineer at (512) 248-6354 during non-business hours.  Phone Confirmation for forced and Level 1 Maintenance Outage schedules is required.

References:

Nodal Protocols

2.1 Definitions

Outage

The condition of a Transmission Facility or Generation Resource that is part of the ERCOT Transmission Grid and defined in the Network Operations Model that has been removed from its normal service, excluding the operation of Transmission Facilities associated with the start-up and shutdown of Generation Resources.

3.1.4.1 Single Point of Contact

- All communications concerning a Planned Outage or Maintenance Outage must be between ERCOT and the designated “Single Point of Contact” for each TSP or Resource Entity. All nonverbal communications concerning Planned Outages must be conveyed through an electronic interface as specified by ERCOT. The TSP or Resource Entity shall identify, in its initial request or response, the Single Point of Contact, with primary and alternate means of communication. The Resource Entity or TSP shall submit a Notice of Change of Information (NCI) form when changes occur to a Single Point of Contact. This identification must be confirmed in all communications with ERCOT regarding Planned Outage or Maintenance Outage requests.

- The Single Point of Contact must be either a person or a position available seven days per week and 24 hours per day for each Resource Entity and TSP. The Resource Entity shall designate its QSE as its Single Point of Contact. The Single Point of Contact for the TSP must be designated under the ERCOT Operating Guides.

3.1.5.1	ERCOT Evaluation of Planned Outage and Maintenance Outage of Transmission Facilities

(1)	TSPs and Resource Entities shall request a Planned Outage or Maintenance Outage for their respective Transmission Facilities (i.e. Transmission Facilities owned by such Entities) that are part of the ERCOT Transmission Grid and defined in the Network Operations Model.   For Resource Entities within a Private Use Network, this only includes Transmission Facilities at the Point of Interconnection (POI).  Specifically, such requests shall be made when a relevant Transmission Facility will be removed from its normal service.  For TSP requests, the TSPs shall enter such requests in the Outage Scheduler.  For Resource Entity requests, ERCOT shall enter such requests in the Outage Scheduler on behalf of the Resource Entity.  ERCOT’s obligation to enter outage requests for Resource Entities is valid only for the purposes of Transmission Facilities Outage Scheduler entry requirements prescribed by the ERCOT Protocols and Operating Guides. ERCOT’s obligations hereunder do not create any legal obligation or responsibility related to any other legal obligations applicable to a Resource Entity.  ERCOT’s obligation shall terminate upon the implementation of system changes that enable Resource Entities to perform this function.  Planned Outages or Maintenance Outages for Electrical Buses will be treated as consequentially outaged Transmission Elements.  In those cases where a TSP enters the breaker and switch statuses associated with an Electrical Bus, a downstream topology processor will evaluate the breakers and switches associated with the applicable Electrical Bus to determine if the Electrical Bus is consequentially outaged, and to thereby designate the status of the Electrical Bus.  Proposed Transmission Planned Outage or Maintenance Outage information submitted by a TSP or Resource Entity in accordance with this Section constitutes a request for ERCOT’s approval of the Outage Schedule associated with the Planned Outage or Maintenance Outage.  ERCOT is not deemed to have approved the Outage Schedule associated with the Planned Outage or Maintenance Outage until ERCOT notifies the TSP or Resource Entity of its approval under procedures adopted by ERCOT.  Once ERCOT posts a Resource Entity Transmission Facility Outage request on the MIS Secure Area, the Resource Entity shall review the submitted Outage for accuracy and request that ERCOT submit modifications if necessary.  ERCOT shall evaluate requests under Section 3.1.5.11, Evaluation of Transmission Facilities Planned Outage or Maintenance Outage Requests.

3.3.2.1	Information to Be Provided to ERCOT

(1)	The energization or removal of a Transmission Facility or Generation Resource in the Network Operations Model requires an entry into the Outage Scheduler by a TSP or Resource Entity.  The Resource Entity shall make entries in the Outage Scheduler for Outages of new or relocated Generation Resources.  ERCOT shall submit entries into the Outage Scheduler on behalf of the Resource Entity, for any energization or removal of Transmission Facilities owned by the Resource Entity.  Once ERCOT posts a Resource Entity Transmission Facility Outage request on the MIS Secure Area, the Resource Entity shall review the submitted Outage for accuracy of information and request  that ERCOT submit modifications if necessary.  If any changes in system topology or telemetry are expected, then the TSP or Resource Entity shall notify ERCOT in accordance with the schedule in Section 3.3.1, ERCOT Approval of New or Relocated Facilities.  Information submitted pursuant to Section 3.3.2.1 for Transmission Facilities within a Private Use Network shall not be publicly posted.

(2)	If a Resource Entity within a Private Use Network is adding or removing a Transmission Facility at the Point of Interconnection (POI), it shall inform and determine with ERCOT whether any corresponding Network Operations Model updates are necessary.  If ERCOT and the Resource Entity determine that updates are needed, the process set forth in paragraph (1) above shall be used to incorporate the update into the Network Operations Model and Outage Scheduler.  Information submitted pursuant to paragraph (1) above shall not be publicly posted.

(3)	TSPs and Resource Entities shall submit any changes in system topology or telemetry in accordance with the Network Operations Model Change Request (NOMCR) process or other ERCOT-prescribed process applicable to Resource Entities and according to the requirements of Section 3.10.1, Time Line for Network Operations Model Changes.  The submittal shall include the following:

(a)	Proposed energize date;

(b)	TSPs, or Resource Entities performing work;

(c)	TSPs, or Resource Entities responsible for rating affected Transmission Element(s);

(d)	For Resource Entities, data and information required by Section 16.5, Registration of a Resource Entity;

(e)	Station identification code;

(f)	Identification of existing Transmission Facilities involved and new Transmission Facilities (if any) being added or existing Transmission Facilities being permanently removed from service;

(g)	Ratings of existing Transmission Facilities involved and new Transmission Facilities (if any) being added;

(h)	Outages required (clearly identify each Outage if multiple Outages are required), including sequence of Outage and estimate of Outage duration;

(i)	General statement of work to be completed with intermediate progress dates and events identified;

(j)	SCADA modification work, including descriptions of the telemetry points or changes to existing telemetry, providing information on equipment being installed, changed, or monitored;

(k)	Additional data determined by ERCOT and TSPs, or Resource Entities as needed to complete the ERCOT model representation of existing Transmission Facilities involved and new Transmission Facilities (if any) being added;

(l)	Statement of completion, including:

(i)	Statement to be made at the completion of each intermediate stage of project; and

(ii)	Statement to be made at completion of total project.

(m)	Drawings, including:

(i)	Existing status;

(ii)	Each intermediate stage; and

(iii)	Proposed final configuration.

[1] All emails of the “Resource Transmission Outage Request Forms” should be verified by a phone call from the QSE to ERCOT.

[2] ERCOT shall act as the responsible Entity that will manage Outage Scheduler entries for Transmission Elements owned by the Resource. This excludes entries of Resources since the Resource Entities QSE has sufficient roles and permission to submit outages for Resources that is acts as the designated point of contact for and can submit the Resource outages into the ERCOT outage scheduler.  ERCOT’s obligations in regard to submitting Transmission Outages for the Resources Entity and QSE shall terminate upon the implementation of Outage Scheduler system changes and protocol changes that enable Resource Entity QSEs to enter Outages of Transmission Facilities in the Outage Scheduler.