ERCOT 2018

Severe Weather Drill

Script for Real Time/Resource Desk (ERCOT Version)

(NOTE- times associated with the events on this script are estimates only)

The beginning of each hotline call shall begin as follows:

“ERCOT is conducting a drill. All of the following instructions are for simulation and test purposes only. No real-time action is to be taken.”

| DAY 1 | 14:00 | COMMUNICATION TESTING and Simulator Login Verification |
| --- | --- | --- |

| DAY 1 | 14:30 | ocn: ISSUED WHEN THE STORM IS FIVE DAYS AWAY |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill. All of the following instructions are for simulation and test purposes only. No real-time action is to be taken.

(Q#58) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. At [xx:xx], ERCOT is issuing an OCN for [Tropical Storm/Hurricane] [Name] due to a probability of making landfall in the ERCOT Region. QSEs are instructed to:

- Review fuel supplies and notify ERCOT of any known or anticipated fuel restrictions,

- Review Planned Resource outages and consider delaying maintenance,

- Review emergency operating procedures and notify ERCOT of any changes or conditions that could affect System Reliability.

ERCOT will continue to monitor the Storm. [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 1 | 15:00 | advisory: issued when storm is three days away |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill. All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.

(Q#59) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. At [xx:xx], ERCOT is issuing an Advisory for the [Tropical Storm/Hurricane] [Name] in the Gulf of Mexico which is currently projected to impact the ERCOT Region. QSEs are instructed to:

- Review fuel supplies and notify ERCOT of any known or anticipated fuel restrictions,

- Review Planned Resource outages and consider delaying maintenance,

- Review emergency operating procedures and notify ERCOT of any changes or conditions that could affect System Reliability.

- Instruct Plant Operators to prepare for projected severe weather conditions and review procedures for operating in the lead due to possible high voltage concerns

- Notify ERCOT if relocating personnel to backup control centers

ERCOT will continue to monitor the Storm [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 1 | 15:30 | watch: issued when storm is one day away |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill.  All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.

(Q#60) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back. This is ERCOT operator [first and last name]. At [xx:xx], ERCOT is issuing a Watch for [Tropical Storm/Hurricane] [Name] in the Gulf of Mexico which is projected to make landfall near [Place] at [time].   QSEs are instructed to:

- Notify all Resources of conditions

- Review fuel supplies and notify ERCOT of any known or anticipated fuel restrictions,

- Review emergency operating procedures and notify ERCOT of any changes or conditions that could affect System Reliability.

- Make available, any Resources that can be returned to service and keep COPs and HSLs updated

ERCOT will continue to monitor the Storm. [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 2 | 08:00 | market notification of media appeal |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill.  All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.

“This is ERCOT operator [first and last name]; ERCOT has issued an appeal through the public news media for voluntary energy conservation.  Any questions? Thank you”.

“This is a drill.”

| DAY 2 | 11:00 | emergency: issued when the storm is in texas |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill.  All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.

(Q#61) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back. This is ERCOT Operator [first and last name]. At [xx:xx], ERCOT is issuing an Emergency Notice due to [Tropical Storm/Hurricane Name] having an adverse impact on the ERCOT System [specify condition].  QSEs are instructed to:

- Be prepared to reduce Generator output due to anticipated load loss and respond to voltage support issues as requested,

- Keep COPs and HSLs current.

ERCOT will continue to monitor [Tropical Storm/Hurricane Name].  [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 2 | 11:30 | advisory issued for prc below 3000 mw |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill.  All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.

(Q#1) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is issuing an Advisory due to Physical Responsive Capability falling below 3000 MW.  [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 2 | 12:00 | watch issued for prc below 2500 mw |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill.  All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.

(Q#3) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is issuing a Watch due to Physical Responsive Capability falling below 2500 MW.    Suspend all Resource testing and update telemetered Resource Status Codes.  ERCOT is reminding you, that you can schedule available interchange into ERCOT across the DC Ties.  [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 2 | 12:30 | eea 1 |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill. All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.”

(Q#5) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back. This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is declaring EEA 1, QSEs are to perform their necessary steps for EEA 1.  [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 2 | 13:00 | eea 2 |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill. All of the following instructions are for simulation and test purposes only. No real-time action is to be taken.”

(Q#8) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name], at [xx:xx], ERCOT is declaring EEA 2 and is issuing an appeal through the public news media for voluntary energy conservation.  [Group 1 / Group 1 and 2] Load Resources have been deployed.  Implement all measures associated with EEA 1, if not already implemented. [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 2 | 13:30 | eea 3 |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill. All of the following instructions are for simulation and test purposes only. No real-time action is to be taken.”

(Q#11) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name], at [xx:xx], ERCOT is declaring EEA 3 with firm load shed, [MW Amount] of firm load is being shed.  Implement all measures associated with EEA 1 and 2, if not already implemented. [QSE] please repeat this back to me.”

“This is a drill”

| DAY 2 | 14:30 | restore firm load |
| --- | --- | --- |

Script: “ERCOT IS CONDUCTING A DRILL. ALL OF THE FOLLOWING INSTRUCTIONS ARE FOR SIMULATION AND TEST PURPOSES ONLY.  NO REAL-TIME ACTION IS TO BE TAKEN”

(Q#13) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name], at [xx:xx], ERCOT is restoring firm load.

[QSE] please repeat this back to me.”

“This is a drill”

| DAY 2 | 15:00 | eea 3 to eea 2 |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill.  All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.”

(Q#14) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is moving from EEA 3 to EEA 2.  QSEs are to recall all Load Resources at this time.  [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 2 | 15:15 | eea 2 to eea 1 |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill.  All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.”

(Q#15) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is moving from EEA 2 to EEA 1.  QSE’s are to recall [10 MIN], [30 MIN], [weather-sensitive ERS], [both 10 MIN and 30 MIN] or [both 10 MIN and 30 MIN including weather-sensitive] ERS Resources at this time. [QSE] please repeat this back to me.”

“This is a drill.”

| DAY 2 | 15:30 | eea 1 to termination |
| --- | --- | --- |

Script:  “ERCOT is conducting a drill.  All of the following instructions are for simulation and test purposes only.  No real-time action is to be taken.”

(Q#16) “This call requires everyone to remain on the line until it is complete.  [QSE] I will be asking you for the repeat back.  This is ERCOT Operator [first and last name].  At [xx:xx], ERCOT is terminating EEA 1.  A Watch still remains in effect.  [QSE] please repeat this back to me.”

“This drill is now concluded.”